SPARKER DC-CDI - mounting instructions for Yamaha TW200

We offer replacement of ignition with name "SPARKER DC-CDI". Ignition SPARKER DC-CDI is microprocessor controlled and is successfully working on many motorbikes. The ignition has same advance as function of rpm (speed) characteristic as the original ignition. Ignition SPARKER DC-CDI isn't supply from voltage generator but from 12 Volts board supply. It needs 12 Volts board supply in time of starting (functional battery).

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DC-CDI has various colors. (see table). Counterpart connectors are included.

| SPARKER DC-CDI | motorbike | note |
| --- | --- | --- |
| blue | white | CKPS |
| yellow | green | CKPS |
| red |  | key switched + 12 V |
| blue | black | ground |
| orange | orange | induction coil |
| green | black / white | kill switch |
| black | blue | switch clutch |
| grey | blue/yellow | side stand switch |

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz