SPARKER DCCDIP - mounting instructions for

Aprilia RS125 2008

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DC-CDI-P has various colors. (see table).

| wire color DC-CDI-P | wire color motorbike | name | description |
| --- | --- | --- | --- |
| orange | white/violet | IC | ignition coil |
| yellow | yellow/blue | CKPS | input for pick-up |
| blue | blue | GND | ground for pick-up |
| blue | grey (in 3pin connector) | GND | ground |
| green |  | INPUT 1 | multifunctional input 1 |
| black | white/blue | INPUT 2 | multifunctional input 2 |
| yellow/green | white/yellow | TACHO | tachometer output |
| red | red/black (in 14pin connector), orange (in 3pin connector) | +12V | supply +12V |
| violet | white | gear shift light | output for exhaust valve elmag |

Interconnect orange/blue (in 3pin connector) with yellow/pink (in 14pin connector) without connection to unit DCCDIP2.

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz