SPARKER DC-CDI-P - mounting instructions for Yamaha DT125LC

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DC-CDI-P has various colors. (see table). Counterpart connectors are included.

| wire color DC-CDI-P | wire color motorbike | name | description |
| --- | --- | --- | --- |
| orange | orange | IC | inductive coil |
| grey |  | BLOCK_A | input 2 trough diode (e.g. for neutral switch) |
| black |  | BLOCK | input 2 (e.g. for sidestand switch) |
| yellow | white/red | CKPS | input for pick-up |
| blue | black | GND | ground |
| blue | white/green | GND | ground for CKPS |
| violet |  | GEAR SHIFT LIGHT | output for gear shift light |
| red | brown from ignition key | + 12 V | supply 12 V |
| green/yellow |  | TACHO | output for tachometer |
| green |  | KILL SWITCH | input 1 (e.g. for kill switch) |
| thin violet | black/white | YPVS | output for YPVS |

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz