SPARKER DCCDIP1 race - mounting instructions for Yamaha TDR 250

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DCCDIP1 race has various colors. (see table).

| wire color DCCDIP1 race | pin no. DCCDIP1 race | wire color motorbike | name | description |
| --- | --- | --- | --- | --- |
| grey/black | 1 | light blue | INPUT 3 | neutral/multifunction input |
| white/black | 2 |  | POT | correction potentiometer (analog input) |
| green/yellow | 3 |  | TACHO | tachometer output |
| blue | 4 |  | 232GND | ground for RS232 (pin 5 in 9pin connector)power ground |
| red | 5 | red/white | SUPPLY +12V | supply +12 V |
| violet | 6 |  | POWER OUT 1 | power out 1 (fuel pump relay) |
| blue/white | 7 |  | POWER OUT 2 | power out 2 (gear shift light) |
| pink | 8 |  | POWER OUT 3 | power out 3 (POWER JET Honda) |
| orange | 9 | orange | IC | ignition coil |
| blue | 10 | black, green/white | GND | power ground |
|  | 11 |  | NC | not connected |
| black | 12 | blue/yellow | INPUT 2 | sidestand/multifunction input 2 |
| green/orange | 13 | black/white | kill switch | not maskable kill switch |
| red/black | 14 |  | AC SUPPLY | main ac supply |
| red/black | 15 |  | AC SUPPLY | main ac supply |
|  | 16 |  | NC | not connected |
| yellow/black | 17 |  | AC SUPPLY | auxiliary ac supply |
|  | 18 |  | NC |  |
| grey/red | 19 |  | INPUT 1 | multifunction input 1 |
|  | 20 |  | NC | not connected |
| white/red | 21 |  | + 5V | supply +5V for sensors |
| black | 22 |  | Comm | RS232 (pin 2 in 9pin connector) |
| yellow | 23 |  | Comm | RS232 (pin 3 in 9pin connector) |
| violet | 24 |  | Comm | RS232 connector (pin 1 in 9pin connector) |
| orange | 25 |  | Comm | RS232 connector (pin 9 in 9pin connector) |
| blue | 26 | white//black | SENSE GND | ground for pickup |
| yellow | 27 | white/green | PICK-UP | pickup |
| white/red | 28 | yellow/blue | + 5V | supply +5V for sensors |
| grey | 29 |  | TPS | throttle position sensor input |
| blue | 30 | white/blue | SENSE GND | ground for sensors |
| white/blue | 31 | white/red | STPS | servomotor position sensor input |
| green | 32 | black/brown | M+ | output for servomotor |
| green | 33 | black/red | M- | output for servomotor |
|  | 34 |  | NC | not connected |

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz