SPARKER DCCDIP2 - mounting instructions for Honda XLV750

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DCCDIP2 has various colors. (see tables). Counterpart connector is included.

| wire color DCCDIP2 | pin no. DCCDIP2 | wire color motorbike | name | description |
| --- | --- | --- | --- | --- |
| 2x orange | 1 | 2x yellow/white | IC 1 | inductive coil 1 |
| blue | 2 | green | GND | power ground |
|  | 3 |  | POWEROUT 1 | power output 1 |
|  | 4 |  | M | output for servomotor |
|  | 5 |  | M | output for servomotor |
|  | 6 |  | TPS | throttle position sensor |
|  | 7 |  |  |  |
|  | 8 |  | INPUT 2 | multifunctional input 2 |
| yellow | 9 | yellow | PICK-UP 1 | input for pick-up 1 |
| 2x white | 10 | 2x blue/white | IC 2 | inductive coil 2 |
| blue | 11 | green | GND | power ground |
|  | 12 |  | POWEROUT 2 | power output 2 |
| red | 13 | black (from ignition key) | + 12 V | supply 12 V |
|  | 14 |  | POWEROUT 3 | power output 3 |
|  | 15 |  | TACHO | output for tachometer |
| 2x blue | 16 |  | SENSE GND | ground for sensors |
|  | 17 |  | + 5 V | supply for sensors |
|  | 18 |  | STPS | servo position sensor |
| 2x grey/red | 19 | 2x black/white | INPUT 1 | multifunctional input 1 |
| brown | 20 | blue | PICK-UP 2 | input for pick-up 2 |

Wires black/yellow and black/blue do not connected or connect it to ground.