SPARKER DCCDIP2 - mounting instructions for Yamaha DT125LC

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DCCDIP2 has various colors. (see table).

| wire color DCCDIP2 | pin no. DCCDIP2 | wire color motorbike | name | description |
| --- | --- | --- | --- | --- |
| orange | 1 | orange | IC 1 | ignition coil 1 |
| blue | 2 | white/green | SENSE GND | ground for sensors |
|  | 3 |  | POWEROUT 1 | poweroutput 1 |
| green | 4 | black/red at servomotor | M | output for servomotor |
| green | 5 | black/brown at servomotor | M | output for servomotor |
|  | 6 |  | TPS | throttle position sensor |
|  | 7 |  |  | reserve |
|  | 8 |  | INPUT 2 | multifunctional input 2 |
| yellow | 9 | white/red | CKPS (1) | input for pick-up 1 |
|  | 10 |  | IC 2 | ignition coil 2 |
| blue | 11 | black | GND | ground |
|  | 12 |  | POWEROUT 2 | poweroutput 2 |
| red | 13 | brown at key switch | + 12 V | supply 12 V |
|  | 14 |  | POWEROUT 3 | poweroutput 3 |
|  | 15 |  | TACHO | output for tachometer |
| blue | 16 | white/black at servomotor | SENSE GND | ground for sensors |
| white/red | 17 | yellow/blue at servomotor | + 5 V | supply for sensors |
| white/blue | 18 | white/red at servomotor | STPS | servo position sensor |
| grey/red | 19 | black/white | INPUT 1 | multifunctional input 1 (used for kill switch) |
|  | 20 |  | CKPS (2) | input for pick-up 2 |

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz