[image: image1.png]lani
gn Tach




SPARKER DCCDIP2 race - mounting instructions for Suzuki RG500
Wiring
Wiring must be done according the table below. The wires of ignition SPARKER DCCDIP2 has various colors. (see table). Counterpart connectors are included.
	color in  cable

adapter
	pin no. in

connector
	wire color

motorbike
	name
	description

	white
	1
	
	IC 2
	ignition coil 2

	violet
	2
	
	POWER OUT 1
	Power out 1 (fuel pump relay)

	blue/white
	3
	
	POWER OUT 2
	Power out 2 (gear shift light)

	green
	4
	
	M
	output for servomotor

	pink
	5
	
	POWER OUT 3
	Power out 3 (solenoid valve 1)

	grey
	6
	
	TPS
	throttle position sensor input

	red
	7
	orange (from ignition key)
	SUPPLY +12V
	supply +12 V

	white/blue
	8
	
	STPS
	servomotor position sensor input

	black
	9
	
	
	RS232 (pin 2 in 9pin connector)

	yellow
	10
	
	
	RS232 (pin 3 in 9pin connector)

	violet/black
	11
	
	POWER OUT 4
	Power out 4 (solenoid valve 2)

	white/black
	12
	
	POT
	correction potentiometer

	yellow
	13
	brown
	PICK-UP 1
	pickup 1

	orange
	14
	white/blue
	IC 1
	ignition coil 1

	blue
	15
	black/white, white
	GND
	power ground

	blue
	16
	
	GND
	ground for RS232 (pin 5 in 9pin connector)

	green
	17
	
	M
	output for servomotor

	orange
	18
	
	
	RS232 connector (pin 9 in 9pin connector)

	green/yellow
	19
	
	TACHO
	tachometer output

	white/red
	20
	
	+ 5V
	supply +5V for sensors

	violet
	21
	
	
	RS232 connector (pin 1 in 9pin connector)

	grey/black
	22
	
	INPUT 3
	multifunction input 3

	light blue
	23
	black
	SENSE GND
	ground for sensors

	black
	24
	
	INPUT 2
	multifunction input 2

	grey/red
	25
	black/yellow
	INPUT 1
	multifunction input 1 (used as KILL switch)

	brown
	26
	
	PICK-UP 2
	pickup 2


Producer address: 
IgniTech
Ceskobratrska 84




535 01 PRELOUC




Czech Republic
ignitech@ignitech.cz 
www.ignitech.cz

