SPARKER TCI - mounting instructions for Kawasaki ZX7R

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER TCI has various colors. (see table). Counterpart connectors are included.

| SPARKER TCI | motorbike | note |
| --- | --- | --- |
| blue | black/white | sensor ground |
| yellow | yellow | CKPS |
| black | green/black | blocking input |
| green/yellow | black/blue | tachometer |
| violet | black/red | fuel pum relay |
| red | brown/white | key switched + 12 V |
| blue | black/yellow | ground |
| orange | black | induction coil 1, 4 |
| white | green | induction coil 2, 3 |

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz