[image: image1.png]lani
gn Tach




SPARKER TCIP4 - mounting instructions for Honda CBR1000RR

2006 - 2007. Replacement from injection to carburators.
Wiring
Wiring must be done according the table below. The wires of ignition SPARKER TCIP4 has various colors. (see table).
	wire color

TCIP4
	pin no.

TCIP4
	wire color

motorbike
	pin no.

original unit
	name
	description

	orange
	1
	blue/black
	12/black con.
	IC 1
	inductive coil 1

	yellow/black
	2
	red/blue
	2/black con.
	IC 3
	inductive coil 3

	violet
	3
	brown/yellow
	21/black con.
	fuel pump relay
	output for fuel pump relay

	green
	4
	red
	10/black con.
	M
	output for servomotor

	green
	5
	blue
	11/black con.
	M
	output for servomotor

	grey
	6
	red/yellow
	31/grey con.
	TPS
	throttle position sensor

	blue
	7
	green/pink
	23/black con.
	GND
	ground

	black
	8
	
	
	INPUT 1
	multifunctional input 1

	yellow
	9
	grey
	33/grey con.
	CKPS (1)
	input for pick-up (1) - used for cam

	white
	10
	blue/red
	1/black con.
	IC 2
	inductive coil 2

	red/black
	11
	red/yellow
	3/black con.
	IC 4
	inductive coil 4

	green/black
	12
	
	
	gear shift light
	output for gear shift light

	red
	13
	black/white
	4, 5//black con.
	+ 12 V
	supply 12 V

	blue
	14
	green/pink
	24/black con.
	GND
	ground

	green/yellow
	15
	yellow/green
	17/grey con.
	TACHO
	output for tachometer

	blue
	16
	white/black (grey/black)
	32 (18)/black con.
	SENSE GND
	ground for sensors

	white/red
	17
	yellow/red
	9/black con.
	+ 5 V
	+5V supply for sensors

	white/blue
	18
	light green/black
	18/grey con.
	STPS
	servo position sensor

	grey/red
	19
	
	
	INPUT 2
	multifunctional input 2

	brown
	20
	yellow
	22/grey con.
	CKPS (2)
	input for pick-up (2) -used for crank


Producer address: 
IgniTech
Chrudimská 1739




535 01 PRELOUC




Czech Republic
ignitech@ignitech.cz 
www.ignitech.cz

