SPARKER DC-CDI - mounting instructions for Husquarna LT610

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DC-CDI has various colors. (see table). Counterpart connectors are included.

| zapalování SPARKER DC-CDI | motocykl | původní zapalování | pozn. |
| --- | --- | --- | --- |
| blue | green | green | Pick-up |
| yellow | white | white | Pick-up |
| red | green/red |  | key switched + 12 V |
| green / yellow | green/grey | blue | Tachometer |
| blue | blue | black/white | ground |
| orange | brown/white | white/blue | Ignition coil |
| green | orange/white | black/yellow | Kill switch |

Producer address: 	IgniTech

Chrudimská 1739

535 01 PRELOUC

ignitech@ignitech.cz

www.ignitech.cz