SPARKER DC-CDI - mounting instructions for Gas Gas EC250

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DC-CDI-P has various colors. (see table).

| wire color DC-CDI-P | wire color motorbike | name | description |
| --- | --- | --- | --- |
| orange | white/blue | IC | inductive coil |
| yellow | green | CKPS | input for pick-up |
| blue | red | GND | ground for pick-up |
| blue | black or black/white | GND | ground |
| green | black/yellow | KILL | input for KILL switch |
| white/red | black/red | AC | AC magneto |
| white/red | red/white | AC | AC magneto |

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz