SPARKER DC-CDI - mounting instructions for

Honda NSR125

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DC-CDI has various colors. (see table). Counterpart connectors are included.

| SPARKER DC-CDI | motorbike | note |
| --- | --- | --- |
| green | black/ white | engine stop switch |
|  | black/red | magneto coil - don´t connect |
| green/yellow | red/white | exhaust valve |
| yellow | blue | CKPS |
| blue | green | ground |
| orange | yellow | induction coil |
| black | green/white | blocking (sidestand or neutral), shur-plug |
| red | black | key switched + 12 V(e.g. on charging rectifier-regulator) |

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz