SPARKER DC-CDI - mounting instructions for

Honda XL600 with starter

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DC-CDI has various colors. (see table). Counterpart connectors are included.

| SPARKER DC-CDI | motorbike | note |
| --- | --- | --- |
| blue | black/red | stator coil |
| yellow | blue/yellow | pickup |
| red | black from ignition key | key switched + 12 V |
| blue | green | ground |
| orange | black/yellow | ignition coil |
| green | black/white | engine stop switch |

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz