SPARKER DC-CDI - mounting instructions for Honda XLV600

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DC-CDI has various colors. (see table). Counterpart connectors are included.

Replacement unit cylinder 1

| SPARKER DC-CDI | motorbike | note |
| --- | --- | --- |
| yellow | blue | CKPS 1 |
| black | green/yellow (both join) | blocking input |
| red | black/white (both join) | key switched + 12 V |
| blue | green | ground |
| orange | black/yellow (both join) | induction coil 1 |
| white | black/blue (both join) | induction coil 2 |

Replacement unit cylinder 2

| SPARKER DC-CDI | motorbike | note |
| --- | --- | --- |
| brown | yellow | CKPS 2 |
| black | green/yellow (both join) | blocking input |
| red | black/white (both join) | key switched + 12 V |
| blue | green | ground |
| orange | black/yellow (both join) | induction coil 1 |
| white | black/blue (both join) | induction coil 2 |

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz