SPARKER DC-CDI - mounting instructions for

Husaberg FE650

(model with SEM ignition)

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DC-CDI has various colors. (see table). Counterpart connectors are included.

| SPARKER DC-CDI | motorbike | note |
| --- | --- | --- |
| blue | black (from magneto) | ground |
| yellow | green (from magneto) | pick-up |
| green | orange | KILL switch |
| red |  | key switched +12 V |
| blue |  | ground for ignition coil |
| orange |  | ignition coil |

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz