SPARKER DC-CDI - mounting instructions for Kawasaki KLR650

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DC-CDI has various colors. (see table). Counterpart connectors are included.

| SPARKER DC-CDI | motorbike | original unit | note |
| --- | --- | --- | --- |
| blue | black/red | blue/red | sensor ground |
| yellow | black (in 2pin connector) | red/green | CKPS |
| black | red/black | red/black | blocking input |
| red | brown/white |  | key switched + 12 V |
| blue | black/yellow (from key switch) | black | ground |
| orange | black (in 6pin connector) | black/white | induction coil |
| green | red | red | magneto coil |
| green | black/white | white | magneto coil, engine stop switch |

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz