SPARKER DC-CDI - mounting instructions for Kawasaki KLX650R

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DC-CDI has various colors. (see table). Counterpart connectors are included.

| SPARKER DC-CDI | motorbike | original unit | note |
| --- | --- | --- | --- |
| blue | green/white | green/white | sensor ground |
| yellow | white/yellow | white/yellow | CKPS |
| red | yellow (from light switch) |  | + 12 V |
| blue | black/yellow | black/yellow | ground |
| orange | blue | blue | induction coil |
| green | black/white | black/white | magneto coil, engine stop switch |

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz