SPARKER DC-CDI - mounting instructions for Suzuki DR350S

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DC-CDI has various colors. (see table). Counterpart connectors are included.

| SPARKER DC-CDI | motorbike | original unit | note |
| --- | --- | --- | --- |
| blue |  | green | sensor ground |
| yellow |  | blue | CKPS |
| black |  | green/red | side stand switch |
| grey |  | blue/red | neutral switch |
| grey |  | blue/black | pilot-light neutral |
| green/yellow |  | white/red | tachometer |
| red | orange |  | key switched + 12 V |
| blue |  | black/white | ground |
| orange |  | white/blue | induction coil |
| green |  | black/yellow | engine stop switch |

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz