SPARKER DC-CDI - mounting instructions for Suzuki DR800

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DC-CDI has various colors. (see table). Counterpart connectors are included.

| SPARKER DC-CDI | motorbike | note |
| --- | --- | --- |
| blue | green | sensor ground |
| yellow | blue/white | CKPS |
| black | green green/yellow (white) | side stand switch blocking of start relay |
| grey | blue blue/black | neutral switch pilot-light neutral |
| green/yellow | black/green | tachometer |
| red | orange (from ignition key) | key switched + 12 V |
| blue | black/white | ground |
| orange | white/blue | induction coil |
| green | black/yellow | engine stop switch |

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz