SPARKER DC-CDI - mounting instructions for Suzuki TSX125

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DC-CDI has various colors. (see table). Counterpart connectors are included.

| SPARKER DC-CDI | motorbike | note |
| --- | --- | --- |
| blue | blue/violet | sensor ground |
| yellow | red/green | CKPS |
| red | orange from ignition key | key switched + 12 V |
| blue | black/white | ground |
| orange | white/blue | induction coil |
| green | black/red from harness, no from magneto | engine stop switch |

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz