SPARKER DC-CDI - mounting instructions for Honda Yamaha Cygnus 125 (II)

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DC-CDI has various colors. (see table). Counterpart connectors are included.

| SPARKER DC-CDI | motorbike | note |
| --- | --- | --- |
| blue | white/blue | sensor ground |
| yellow | white/red | CKPS |
| red | red/white | key switched + 12 V |
| blue | black/white | ground |
| orange | orange | induction coil |
| black | black | blocking (side stand) |
| red | yellow/red | AUTO CHOKE and HEADLIGHTS RELAY |
| red | yellow/black | HOT GRIP RELAY |

Attention !!!

In this wiring AUTO CHOKE, HEAD LIGHTS RELAY and HOT GRIP RELAY are running immediately if ignition key is on !!!

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz