SPARKER DC-CDI - mounting instructions for Yamaha DT125LC

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DC-CDI has various colors. (see table). Counterpart connectors are included.

| SPARKER DC-CDI | motorbike | note |
| --- | --- | --- |
| blue | white / red | pick-up |
| yellow | white / green | pick-up |
| red | brown from ignition key | key switched + 12 V |
| blue | black | ground |
| orange | orange | ignition HT coil |
| violet | black / white | control of exhaust throttle, kill switch |

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz