SPARKER DC-CDI - mounting instructions for Yamaha DT125R 4BL

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DC-CDI has various colors. (see table). Counterpart connectors are included.

| SPARKER DC-CDI | motorbike | note |
| --- | --- | --- |
| blue | black | sensor ground |
| yellow | white/red | CKPS |
| grey | light blue | neutral switch |
| grey | blue | neutral light |
| black | blue/yellow | side stand switch |
| red | brown from ignition key | key switched + 12 V |
| blue | black | ground |
| orange | orange | induction coil |
| green | black/white | KILL switch |
| violet | black/yellow | control of exhaust throttle, kill switch |

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz