SPARKER DC-CDI - mounting instructions for Honda Yamaha DT200R

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DC-CDI has various colors. (see table). Counterpart connectors are included.

| SPARKER DC-CDI | motorbike | note |
| --- | --- | --- |
| blue | white / green | sensor ground |
| yellow | white / red | CKPS |
| red |  | key switched + 12 V |
| blue | black | ground |
| orange | orange | induction coil |
| green | black / white | kill switch |
| violet | black / yellow | control of exhaust throttle |

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz