SPARKER DC-CDI - mounting instructions for Honda Yamaha XT125

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DC-CDI has various colors. (see table). Counterpart connectors are included.

| SPARKER DC-CDI | original ignition | note |
| --- | --- | --- |
| blue | green | sensor ground |
| yellow | white | CKPS |
| red |  | key switched + 12 V |
| blue | black | ground |
| orange | orange | induction coil |
| green | black / white | engine stop switch |
| green | black / white | engine stop switch |

residual wires (red and brown) don´t connect !!!

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz