SPARKER DC-CDI - mounting instructions for Yamaha XT600

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DC-CDI has various colors. (see table). Counterpart connectors are included.

XT600 (1984-86), XT600Z TÉNÉRÉ (1983-85)

| SPARKER DC-CDI | motorbike | note |
| --- | --- | --- |
| blue | green | ground for CKPS |
| brown | white/red | CKPS 1 |
| yellow | white/ green | CKPS 2 |
| red |  | key switched + 12 V |
| blue | black | ground |
| orange | orange | induction coil |
| green | black/ white | engine stop switch |

XT600 with side stand (from 1987)

| SPARKER DC-CDI | motorbike | note |
| --- | --- | --- |
| blue | black/yellow | ground for CKPS |
| brown | blue/yellow | CKPS 1 |
| yellow | green/white | CKPS 2 |
| red |  | key switched + 12 V |
| grey | light blue | blocking input (NEUTRAL) |
| orange | orange | induction coil |
| green | black/ white | engine stop switch |
| blue | black | ground |
| grey | blue | NEUTRAL light |
| black | blue/yellow | blocking input (side stand) |

XT600Z TÉNÉRÉ with starter (1986-87)

| SPARKER DC-CDI | motorbike | note |
| --- | --- | --- |
| blue | green | ground for CKPS |
| brown | red | CKPS 1 |
| yellow | green/white | CKPS 2 |
| red |  | key switched + 12 V |
| blue | black | ground |
| orange | orange | induction coil |
| green | black/ white | engine stop switch |
| black | blue | blocking input |
| grey | blue/yellow | blocking input |

XT600Z (U) TÉNÉRÉ with starter (from 1988)

| SPARKER DC-CDI | motorbike | note |
| --- | --- | --- |
| blue | black/yellow | ground for CKPS |
| brown | blue/yellow | CKPS 1 |
| yellow | green/white | CKPS 2 |
| red |  | key switched + 12 V |
| blue | black | ground |
| orange | orange | induction coil |
| green | black/ white | engine stop switch |
| black | blue | blocking input |
| grey | blue/yellow | blocking input |

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz