SPARKER DC-CDI - mounting instructions for quad Yamaha YFM350 Warrior

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DC-CDI has various colors. (see table). Counterpart connectors are included.

1)

| SPARKER DC-CDI | motorbike | note |
| --- | --- | --- |
| blue | blue | pickup |
| yellow | yellow | pickup |
| black | black/yellow | clutch switch (not block, for start enable only) |
| black | yellow/black | start enable |
| grey | white/black | neutral switch (not block, for start enable only) |
| red | brown | key switched +12V |
| blue | black | ground |
| blue with splice | black | ground |
| orange | orange | induction coil |
| green | black / white | engine stop switch |

2) version with sealed  Sumitomo and Furukawa connectors

| SPARKER DC-CDI | motorbike | note |
| --- | --- | --- |
| blue | white | pickup |
| yellow | red | pickup |
| black | black/yellow | clutch switch (not block, for start enable only) |
| black | yellow/black | start enable |
| grey | white/black | neutral switch (not block, for start enable only) |
| red | brown | key switched +12V |
| blue | black | ground |
| blue with splice | black | ground |
| orange | orange | induction coil |
| green | black / white | engine stop switch |

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz