SPARKER DCCDIP2 - mounting instructions for Suzuki RGV250

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DCCDIP2 has various colors. (see tables). Counterpart connectors are included.

Our unit have no output for carburator solenoids. If is possible - you can use GEAR SHIFT LIGHT and POWER JET output for control of this solenoids.

You must set working area of TPS (0 and 100 %) and servo of exhaust valve.

K type

| color in cable adapter | pin no. in connector | wire color motorbike | name | description |
| --- | --- | --- | --- | --- |
| white | 1 | white/blue | IC 2 | ignition coil 2 |
| violet | 2 |  | POWER OUT 1 | Power out 1 (fuel pump relay) |
| blue/white | 3 |  | POWER OUT 2 | Power out 2 (gear shift light) |
| green | 4 |  | M | output for servomotor |
| pink | 5 |  | POWER OUT 3 | Power out 3 (POWER JET Honda) |
| grey | 6 |  | TPS | throttle position sensor input |
| red | 7 | orange/black | SUPPLY +12V | supply +12 V |
| white/blue | 8 |  | STPS | servomotor position sensor input |
| black | 9 |  |  | RS232 (pin 2 in 9pin connector) |
| yellow | 10 |  |  | RS232 (pin 3 in 9pin connector) |
| violet/black | 11 |  | POWER OUT 4 | Power out 4 |
| white/black | 12 |  | POT | correction potentiometer |
| yellow | 13 | red | PICK-UP 1 | pickup 1 |
| orange | 14 | black/yellow | IC 1 | ignition coil 1 |
| blue | 15 | black/white | GND | power ground |
| blue | 16 |  | GND | ground for RS232 (pin 5 in 9pin connector) |
| green | 17 |  | M | output for servomotor |
| orange | 18 |  |  | RS232 connector (pin 9 in 9pin connector) |
| green/yellow | 19 |  | TACHO | tachometer output |
| white/red | 20 |  | + 5V | supply +5V for sensors |
| violet | 21 |  |  | RS232 connector (pin 1 in 9pin connector) |
| grey/black | 22 |  | INPUT 3 | multifunction input 3 |
| light blue | 23 | white (from pickups) black (from pickups) | SENSE GND | ground for sensors |
| black | 24 |  | INPUT 2 | multifunction input 2 |
| grey/red | 25 |  | INPUT 1 | multifunction input 1 |
| brown | 26 | brown | PICK-UP 2 | pickup 2 |

L type (replace both original units - remove original CDI unit is necessary)

| color in cable adapter | pin no. in connector | wire color motorbike | name | description |
| --- | --- | --- | --- | --- |
| white | 1 | white/blue (direct to IC) | IC 2 | ignition coil 2 |
| violet | 2 |  | POWER OUT 1 | Power out 1 (fuel pump relay) |
| blue/white | 3 |  | POWER OUT 2 | Power out 2 (gear shift light) |
| green | 4 | black/red | M | output for servomotor |
| pink | 5 |  | POWER OUT 3 | Power out 3 (POWER JET Honda) |
| grey | 6 | blue (from TPS) | TPS | throttle position sensor input |
| red | 7 | orange/black | SUPPLY +12V | supply +12 V |
| white/blue | 8 | yellow | STPS | servomotor position sensor input |
| black | 9 |  |  | RS232 (pin 2 in 9pin connector) |
| yellow | 10 |  |  | RS232 (pin 3 in 9pin connector) |
| violet/black | 11 |  | POWER OUT 4 | Power out 4 |
| white/black | 12 |  | POT | correction potentiometer |
| yellow | 13 | red | PICK-UP 1 | pickup 1 |
| orange | 14 | black/yellow (direct to IC) | IC 1 | ignition coil 1 |
| blue | 15 | black/white | GND | power ground |
| blue | 16 |  | GND | ground for RS232 (pin 5 in 9pin connector) |
| green | 17 | red/black | M | output for servomotor |
| orange | 18 |  |  | RS232 connector (pin 9 in 9pin connector) |
| green/yellow | 19 |  | TACHO | tachometer output |
| white/red | 20 | orange (from STPS) orange (from TPS) | + 5V | supply +5V for sensors |
| violet | 21 |  |  | RS232 connector (pin 1 in 9pin connector) |
| grey/black | 22 |  | INPUT 3 | multifunction input 3 |
| light blue | 23 | white (from pickups) black (from pickups) black/white (from TPS) black/white (from STPS) | SENSE GND | ground for sensors |
| black | 24 |  | INPUT 2 | multifunction input 2 |
| grey/red | 25 |  | INPUT 1 | multifunction input 1 |
| brown | 26 | brown | PICK-UP 2 | pickup 2 |

M type (replace both original units - remove original CDI unit is necessary, without solenoids drive)

| color in cable adapter | pin no. in connector | wire color motorbike | name | description |
| --- | --- | --- | --- | --- |
| white | 1 | white/blue (direct to IC) | IC 2 | ignition coil 2 |
| violet | 2 |  | POWER OUT 1 | Power out 1 (fuel pump relay) |
| blue/white | 3 |  | POWER OUT 2 | Power out 2 (gear shift light) |
| green | 4 | black/red | M | output for servomotor |
| pink | 5 |  | POWER OUT 3 | Power out 3 (POWER JET Honda) |
| grey | 6 | blue (from TPS) | TPS | throttle position sensor input |
| red | 7 | orange/black | SUPPLY +12V | supply +12 V |
| white/blue | 8 | yellow | STPS | servomotor position sensor input |
| black | 9 |  |  | RS232 (pin 2 in 9pin connector) |
| yellow | 10 |  |  | RS232 (pin 3 in 9pin connector) |
| violet/black | 11 |  | POWER OUT 4 | Power out 4 |
| white/black | 12 |  | POT | correction potentiometer |
| yellow | 13 | red | PICK-UP 1 | pickup 1 |
| orange | 14 | black/yellow (direct to IC) | IC 1 | ignition coil 1 |
| blue | 15 | black/white | GND | power ground |
| blue | 16 |  | GND | ground for RS232 (pin 5 in 9pin connector) |
| green | 17 | red/black | M | output for servomotor |
| orange | 18 |  |  | RS232 connector (pin 9 in 9pin connector) |
| green/yellow | 19 | white/red | TACHO | tachometer output |
| white/red | 20 | orange/white | + 5V | supply +5V for sensors |
| violet | 21 |  |  | RS232 connector (pin 1 in 9pin connector) |
| grey/black | 22 |  | INPUT 3 | multifunction input 3 |
| light blue | 23 | white (from pickups) black/green | SENSE GND | ground for sensors |
| black | 24 |  | INPUT 2 | multifunction input 2 |
| grey/red | 25 |  | INPUT 1 | multifunction input 1 |
| brown | 26 | brown | PICK-UP 2 | pickup 2 |

approximately 1996 (replace both original units - remove original CDI unit is necessary, without solenoids drive)

| color in cable adapter | pin no. in connector | wire color motorbike | name | description |
| --- | --- | --- | --- | --- |
| white | 1 | white/blue (direct to IC) | IC 2 | ignition coil 2 |
| violet | 2 |  | POWER OUT 1 | Power out 1 (fuel pump relay) |
| blue/white | 3 |  | POWER OUT 2 | Power out 2 (gear shift light) |
| green | 4 | red/grey | M | output for servomotor |
| pink | 5 |  | POWER OUT 3 | Power out 3 (POWER JET Honda) |
| grey | 6 | blue (from TPS) | TPS | throttle position sensor input |
| red | 7 | orange/black | SUPPLY +12V | supply +12 V |
| white/blue | 8 | yellow | STPS | servomotor position sensor input |
| black | 9 |  |  | RS232 (pin 2 in 9pin connector) |
| yellow | 10 |  |  | RS232 (pin 3 in 9pin connector) |
| violet/black | 11 |  | POWER OUT 4 | Power out 4 |
| white/black | 12 |  | POT | correction potentiometer |
| yellow | 13 | red | PICK-UP 1 | pickup 1 |
| orange | 14 | black/yellow (direct to IC) | IC 1 | ignition coil 1 |
| blue | 15 | white/black (rather black/white) | GND | power ground |
| blue | 16 |  | GND | ground for RS232 (pin 5 in 9pin connector) |
| green | 17 | red/black | M | output for servomotor |
| orange | 18 |  |  | RS232 connector (pin 9 in 9pin connector) |
| green/yellow | 19 |  | TACHO | tachometer output |
| white/red | 20 | orange | + 5V | supply +5V for sensors |
| violet | 21 |  |  | RS232 connector (pin 1 in 9pin connector) |
| grey/black | 22 |  | INPUT 3 | multifunction input 3 |
| light blue | 23 | white (from pickups) green/black (rather black/green) | SENSE GND | ground for sensors |
| black | 24 |  | INPUT 2 | multifunction input 2 |
| grey/red | 25 |  | INPUT 1 | multifunction input 1 |
| brown | 26 | brown | PICK-UP 2 | pickup 2 |

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz