SPARKER DCCDIP2 - mounting instructions for Yamaha TZ125

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DCCDIP2 has various colors. (see table). Counterpart connectors are included.

| color in cable adapter | pin no. in connector | wire color motorbike | name | description |
| --- | --- | --- | --- | --- |
| white | 1 |  | IC 2 | ignition coil 2 |
| violet | 2 |  | POWER OUT 1 | Power out 1 (fuel pump relay) |
| blue/white | 3 |  | POWER OUT 2 | Power out 2 (gear shift light) |
| green | 4 | black/red | M | output for servomotor |
| pink | 5 |  | POWER OUT 3 | Power out 3 (solenoid valve 1) |
| grey | 6 |  | TPS | throttle position sensor input |
| red | 7 | red | SUPPLY +12V | supply +12 V |
| white/blue | 8 | white/red | STPS | servomotor position sensor input |
| black | 9 |  |  | RS232 (pin 2 in 9pin connector) |
| yellow | 10 |  |  | RS232 (pin 3 in 9pin connector) |
| violet/black | 11 | pink | POWER OUT 4 | Power out 4 (solenoid valve 2) |
| white/black | 12 |  | POT | correction potentiometer |
| yellow | 13 | white/green | PICK-UP 1 | pickup 1 |
| orange | 14 | grey | IC 1 | ignition coil 1 |
| blue | 15 | black | GND | power ground |
| blue | 16 |  | GND | ground for RS232 (pin 5 in 9pin connector) |
| green | 17 | black/brown | M | output for servomotor |
| orange | 18 |  |  | RS232 connector (pin 9 in 9pin connector) |
| green/yellow | 19 |  | TACHO | tachometer output |
| white/red | 20 | yellow/blue | + 5V | supply +5V for sensors |
| violet | 21 |  |  | RS232 connector (pin 1 in 9pin connector) |
| grey/black | 22 |  | INPUT 3 | multifunction input 3 |
| light blue | 23 | white/black | SENSE GND | ground for sensors |
| black | 24 |  | INPUT 2 | multifunction input 2 |
| grey/red | 25 | black/white | INPUT 1 | multifunction input 1 (KILL SWITCH) |
| brown | 26 |  | PICK-UP 2 | pickup 2 |

1) we don´t know polarity of pickup

2) we don´t know competency of  solenoid valve (pink)

3) we don´t know data for advance

4) we don´t know data for exhaust valve

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz