SPARKER DCCDIP2 - mounting instructions for special

engine Honda NTV650 - harness XLV600

Wiring

Wiring must be done according the table below. The wires of ignition SPARKER DCCDIP2 has various colors. (see table). Counterpart connectors are included.

| pin no. in connector | wire color motorbike | name | description |
| --- | --- | --- | --- |
| 1 | 2x black/blue | IC 2 | ignition coil 2 |
| 2 |  | POWER OUT 1 | Power out 1 (fuel pump relay) |
| 3 |  | POWER OUT 2 | Power out 2 (gear shift light) |
| 4 |  | M | output for servomotor |
| 5 |  | POWER OUT 3 | Power out 3 (solenoid valve 1) |
| 6 |  | TPS | throttle position sensor input |
| 7 | blac/white | SUPPLY +12V | supply +12 V |
| 8 |  | STPS | servomotor position sensor input |
| 9 |  |  | RS232 (pin 2 in 9pin connector) |
| 10 |  |  | RS232 (pin 3 in 9pin connector) |
| 11 |  | POWER OUT 4 | Power out 4 (solenoid valve 2) |
| 12 |  | POT | correction potentiometer |
| 13 | yellow | PICK-UP 1 | pickup 1 |
| 14 | 2x black/yellow | IC 1 | ignition coil 1 |
| 15 | green | GND | power ground |
| 16 |  | GND | ground for RS232 (pin 5 in 9pin connector) |
| 17 |  | M | output for servomotor |
| 18 |  |  | RS232 connector (pin 9 in 9pin connector) |
| 19 |  | TACHO | tachometer output |
| 20 |  | + 5V | supply +5V for sensors |
| 21 |  |  | RS232 connector (pin 1 in 9pin connector) |
| 22 |  | INPUT 3 | multifunction input 3 |
| 23 | white/yello, white/blue | SENSE GND | ground for sensors |
| 24 |  | INPUT 2 | multifunction input 2 |
| 25 |  | INPUT 1 | multifunction input 1 (KILL SWITCH) |
| 26 | blue | PICK-UP 2 | pickup 2 |

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz